SW-620-TX-W

Application Programming Interface

| Document Revision | v1.8 |
| --- | --- |
| Document Date | January 2026 |
| Supported Firmware | Refer to supported product firmware/software for details |

Contents

0. Device Communication	6

0.1 Overview	6

0.2 Network Connection	6

0.3 Serial Connection	6

0.4 Command Delimiters for Sent Commands	6

1. Device Management	7

1.1 Set Device Alias	7

1.2 Get Device Alias	7

1.3 Get Device Version Information	8

1.4 Device Reboot	8

1.5 Device Reset	9

1.6 Show OSD Elements	9

1.7 Set OSD Element Display	10

1.8 Get OSD Element Display	10

1.9 Get MAC Address	11

1.10 Get Device Serial Number	11

2. Input/Output Settings	12

2.1 Set Output Resolution	12

2.2 Get Output Resolution	12

2.3 Set Input Interface HDCP	13

2.4 Get Input Interface HDCP	13

2.5 Get Input Signal HDCP Status	14

2.6 Set Output Interface HDCP	14

2.7 Get Output Interface HDCP	15

2.8 Get Output Signal HDCP Status	15

2.9 Get Input Connection Status	16

2.10 Get Input Signal Status	16

2.11 Get Input Video Format	17

2.12 Get Input Audio Format	17

2.13 Set Output Volume	18

2.14 Get Output Volume	18

2.15 Set Output Mute	18

2.16 Get Output Mute	19

2.17 Get Output Connection Status	19

2.18 Set MST	20

2.19 Get MST	20

3. Video Switching	21

3.1 Set Automatic Switching	21

3.2 Get Automatic Switching Status	21

3.3 Set Multi-View Function	22

3.4 Get Multi-View Function Status	22

3.5 Switch Input to All Outputs	23

3.6 Switch Input to an Output	23

3.7 Display GUIDE Screen	24

3.8 Get Display Status	24

3.9 Set Current Layout	25

3.10 Set Current Layout With ID	25

3.11 Get Current Layout ID and Name	25

3.12 Set Layout and Video Sources	26

3.13 Get Layout and Video Sources	26

3.14 Get Video Source of a Specified Window	27

4. Audio and Other Signal Switching	27

4.1 Set Audio Auto-Switching	27

4.2 Get Audio Auto-Switch Status	28

4.3 Switch to Wired Audio Source	28

4.4 Switch to BYOD Audio Source	28

4.5 Get Current Audio Source	29

4.6 Set USB Auto-Switch	29

4.7 Get USB Automatic Switch Status	30

4.8 Set USB Host	30

4.9 Get USB Host	30

5. Sleep/Standby and Peripheral Control	31

5.1 Set Automatic Standby	31

5.2 Get Automatic Standby	31

5.3 Set Automatic Standby Timeout	32

5.4 Get Auto-Standby Timeout	32

5.5 Set Device Power On or Standby	32

5.6 Get Device Standby	33

5.7 Send CEC Power On/Off Command	33

5.8 Set CEC Power On/Off Command	34

5.9 Get CEC Power On/Off Command	34

5.10 Get CEC Commands	35

5.11 Get All CEC Commands	35

5.12 Send CEC Command String	36

5.13 Set UART Parameters	36

5.14 Get UART Parameters	36

5.15 Set UART Baud Rate	37

5.16 Get UART Baud Rate	37

5.17 Set UART Command	37

5.18 Delete UART Command	38

5.19 Get UART Commands	38

5.20 Send UART Command	39

5.21 Send UART Command String	39

5.22 Set UART Power On/Off Function	40

5.23 Get UART Power On/Off Function	40

5.24 Send CEC and/or RS232 Commands	41

5.25 Set UART Operating Mode	41

5.26 Get UART Operating Mode	41

5.27 Get MUTE Input	42

6. Network and BYOD Related Settings	42

6.1 Set Network Isolation	42

6.2 Get Network Isolation Settings	43

6.3 Set IP Address - DHCP	43

6.4 Set IP Address - Static	44

6.5 Get IP Address	44

6.6 Configure USB-C Network Adapter Function	45

6.7 Get USB Network Adapter Switch Status	45

6.8 Configure HTTPS	46

6.9 Get HTTPS Switch Status	46

6.10 Set Telnet over TLS	46

6.11 Get Telnet over TLS Status	47

6.12 Set Wi-Fi Operating Mode	47

6.13 Get Wi‑Fi Operating Mode	47

6.14 Set Wi-Fi Password	48

6.15 Get Wi-Fi Password	48

6.16 Configure Soft AP Function	48

6.17 Get Soft AP On/Off Status	49

6.18 Configure SoftAP Routing	49

6.19 Get Soft Router Function Switch Status	49

6.20 Set BYOD Feature	50

6.21 Get BYOD Feature	50

6.22 Set BYOD Access Code	50

6.23 Get BYOD Access Code	51

6.24 Set SSID Broadcast	51

6.25 Get SSID Broadcast Switch Status	51

# 0. Device Communication

## 0.1 Overview

The following contains the connection and commands to control the SW-620-TX-W. By following the content contained in this document, the device can be controlled and configured via 3rd party control system.

IMPORTANT NOTE: Due to differences between WyreStorm device model versions within a series, some commands have different parameters based on the model and version. These differences are noted where applicable and should be followed, as sending an incorrect parameter may cause the unit to lock up and become inoperable.

## 0.2 Network Connection

By default, the IP address is assigned by the DHCP server. If the DHCP server is not available the unit will use a link local IP address based on its MAC address, if the unit was not already configured via the Web interface.

Default communication port number is 23.

## 0.3 Serial Connection

The SW-620-TX-W uses a 3-pin RS-232 with no hardware flow control. Most control systems and computers are DTE where pin 2 is RX, this can vary from device to device. Refer to the documentation for the connected device for pin functionally to ensure that the correct connections can be made.

RS-232 Settings

Baud Rate: 9600

Data Bits: 8bits

Parity: None

Stop Bits: 1bit

Flow Control: None

| WyreStorm Connector |  |  | 3rd Party device |
| --- | --- | --- | --- |
| Pin 1 | 12V DC Out | No Connection | Reserved |
| Pin 2 | TX (Transmit) | ---> To ---> | RX (Receive) |
| Pin 3 | RX (Receive) | ---> To ---> | TX (Transmit) |
| Pin 4 | G (Ground) | ---> To ---> | G (Ground) |

## 0.4 Command Delimiters for Sent Commands

All commands should be delimited with carriage return and line feed.

Accepted delimiter characters are:

| Character | Shorthand | Hex Notation | Escape Notation | Decimal Notation |
| --- | --- | --- | --- | --- |
| Line Feed | LF | 0A | \n | 10 |
| Carriage Return + Line Feed | CR LF | 0D 0A | \r\n | 13 10 |

Please note, most 3rd party control software will either append these characters automatically or an option to specify them will be present. It is important that the last delimited character is LF and not CR.

# 1. Device Management

## 1.1 Set Device Alias

Set the device alias.
This alias is used as the receiver name for BYOD casting methods such as Miracast and AirPlay, and is also used as the SSID for the built-in soft AP.

| Command Structure SET ALIAS < DeivceName > | Parameters: DeviceName = The alias to set, up to 31 characters. Allowed characters: letters, digits, spaces, underscore ('_'), and hyphen ('-'). Spaces cannot appear at the beginning or end. |
| --- | --- |
| Response Structure ALIAS < DeivceName > |  |
| Command Example |  |
| SET ALIAS MeetingRoom |  |
| Response Example |  |
| ALIAS MeetingRoom |  |
| Additional Notes: Default value: SW-640L-TX-W-XXXX, where XXXX are the last two bytes of the MAC address in hexadecimal. For example: SW-640L-TX-W-327C |  |

## 1.2 Get Device Alias

| Command Structure GET ALIAS | Parameters: |
| --- | --- |
| Response Structure ALIAS < DeivceName > |  |
| Command Example |  |
| GET ALIAS |  |
| Response Example |  |
| ALIAS MeetingRoom |  |
| Additional Notes: |  |

## 1.3 Get Device Version Information

Retrieve the version information for each software module on the device.

| Command Structure GET VER < ALL ¦ ModuleName > | Parameters: ModuleName = the software module whose version you want to retrieve. Currently supported values: MAINSOC, USBCCHIP, USBCVIDEO, VIDEOCHIP ALL = retrieve version information for all software modules; in this case the device returns each module's version on separate lines. |
| --- | --- |
| Response Structure VER ModuleName Version or VER ModuleName1 Version1 VER ModuleName2 Version2 ... |  |
| Command Example |  |
| GET VER MAINSOC |  |
| Response Example |  |
| VER MAINSOC 1.4.0 |  |
| Additional Notes: |  |

## 1.4 Device Reboot

Sends a command to reboot the device.

| Command Structure REBOOT | Parameters: |
| --- | --- |
| Response Structure REBOOT |  |
| Command Example |  |
| REBOOT |  |
| Response Example |  |
| REBOOT |  |
| Additional Notes: |  |

## 1.5 Device Reset

Command to restore the device to factory default settings.

| Command Structure RESET [ GUIDE ] | Parameters: If the optional parameter GUIDE is included, only the image on the Guide screen will be reset. |
| --- | --- |
| Response Structure RESET [ GUIDE ] |  |
| Command Example |  |
| RESET |  |
| Response Example |  |
| RESET |  |
| Additional Notes: |  |

## 1.6 Show OSD Elements

Display all allowed OSD elements for 10 seconds.

| Command Structure SET SHOWOSD | Parameters: |
| --- | --- |
| Response Structure SHOWOSD |  |
| Command Example |  |
| SET SHOWOSD |  |
| Response Example |  |
| SHOWOSD |  |
| Additional Notes: |  |

## 1.7 Set OSD Element Display

Configure whether a specific OSD element is allowed to be displayed

| Command Structure SET SHOWOSD < DEVICENAME ¦ IPADDRESS ¦ WIFIPWD ¦ ACCESSCODE > < ON ¦ OFF > | Parameters: For the 1st parameter, the four listed values correspond to device name, IP address, Wi‑Fi password, and access code, respectively. |
| --- | --- |
| Response Structure SHOWOSD < DEVICENAME ¦ IPADDRESS ¦ WIFIPWD ¦ ACCESSCODE > < ON ¦ OFF > |  |
| Command Example |  |
| SET SHOWOSD DEVICENAME OFF |  |
| Response Example |  |
| SHOWOSD DEVICENAME OFF |  |
| Additional Notes: Default: All four OSD elements mentioned are allowed to be displayed (default value ON). |  |

## 1.8 Get OSD Element Display

Get the configuration that indicates whether OSD elements are allowed to be displayed.

| Command Structure GET SHOWOSD < DEVICENAME ¦ IPADDRESS ¦ WIFIPWD ¦ ACCESSCODE ¦ ALL > | Parameters: ALL = Retrieve display settings for all OSD elements; the device will return each OSD element's display setting line by line. |
| --- | --- |
| Response Structure SHOWOSD < DEVICENAME ¦ IPADDRESS ¦ WIFIPWD ¦ ACCESSCODE > < ON ¦ OFF > or SHOWOSD DEVICENAME < ON ¦ OFF > SHOWOSD IPADDRESS < ON ¦ OFF > SHOWOSD WIFIPWD < ON ¦ OFF > SHOWOSD ACCESSCODE < ON ¦ OFF > |  |
| Command Example |  |
| GET SHOWOSD DEVICENAME |  |
| Response Example |  |
| SHOWOSD DEVICENAME OFF |  |
| Additional Notes: |  |

## 1.9 Get MAC Address

Get the device's MAC address information.

| Command Structure GET MACADDR < CONTROL ¦ SERVICE ¦ ALL > | Parameters: CONTROL = Get the MAC address of the control network port SERVICE = Get the MAC address of the service network port ALL = Get the MAC addresses of both ports above |
| --- | --- |
| Response Structure MACADDR < CONTROL ¦ SERVICE > < MACAddress > or MACADDR CONTROL < MACAddress0 > MACADDR SERVICE < MACAddress1 > |  |
| Command Example |  |
| GET MACADDR CONTROL |  |
| Response Example |  |
| MACADDR CONTROL E4:CE:02:13:5F:A1 |  |
| Additional Notes: |  |

## 1.10 Get Device Serial Number

Read the device serial number.

| Command Structure GET SYS_SN | Parameters: |
| --- | --- |
| Response Structure SYS_SN < SerialNumber > |  |
| Command Example |  |
| GET SYS_SN |  |
| Response Example |  |
| SYN_SN WS29125260001 |  |
| Additional Notes: |  |

# 2. Input/Output Settings

## 2.1 Set Output Resolution

Set the resolution for the HDMI output interface. You can specify a specific resolution directly, or let the device automatically determine the resolution based on the connected display.

| Command Structure SET VIDOUT_RES < OUT1 ¦ OUT2 > < AUTO ¦ Resolution > | Parameters: AUTO = Automatically select the best output resolution based on the connected display's EDID. Resolution = A specific resolution. Valid values are: 3840x2160P@60 3840x2160P@50 3840x2160P@30 3840x2160P@25 3840x2160P@24 1920x1080P@60 1920x1080P@50 1920x1080P@30 1920x1080P@25 1920x1080P@24 1680x1050P@60 1600x1200P@60 1440x900P@60 1366x768P@60 1280x1024P@60 1280x720P@60 1280x720P@50 1024x768P@60 800x600P@60 720x480P@60 640x480P@60 Note: 3840x2160P@60 and 3840x2160P@50 are only available for OUT1. |
| --- | --- |
| Response Structure VIDOUT_RES < OUT1 ¦ OUT2 > < AUTO ¦ Resolution > |  |
| Command Example |  |
| SET VIDOUT_RES OUT1 AUTO |  |
| Response Example |  |
| VIDOUT_RES OUT1 AUTO |  |
| Additional Notes: Default: Both outputs are set to AUTO |  |

## 2.2 Get Output Resolution

Retrieve the resolution of the HDMI output ports.

| Command Structure GET VIDOUT_RES < OUT1 ¦ OUT2 ¦ ALL > | Parameters: ALL = Get the resolutions of both output ports; the device will return the two output resolutions on two separate lines. AUTO = The output port is set to automatic resolution. DISCONNECTED = The output port is set to automatic resolution but currently has no display connected. |
| --- | --- |
| Response Structure VIDOUT_RES < OUT1 ¦ OUT2 > < Resolution ¦ DISCONNECTED > [ AUTO ] or VIDOUT_RES OUT1 < Resolution ¦ DISCONNECTED > [ AUTO ] VIDOUT_RES OUT2 < Resolution ¦ DISCONNECTED > [ AUTO ] |  |
| Command Example |  |
| GET VIDOUT_RES OUT1 |  |
| Response Example |  |
| VIDOUT_RES OUT1 3840x2160P@60 AUTO |  |
| Additional Notes: |  |

## 2.3 Set Input Interface HDCP

Enable or disable HDCP for an input interface.

| Command Structure SET HDCP_S < IN1 ¦ IN2 > < ON ¦ OFF > | Parameters: |
| --- | --- |
| Response Structure HDCP_S < IN1 ¦ IN2 > < ON ¦ OFF > |  |
| Command Example |  |
| SET HDCP_S IN1 OFF |  |
| Response Example |  |
| HDCP_S IN1 OFF |  |
| Additional Notes: Default: HDCP is enabled on all input interfaces. |  |

## 2.4 Get Input Interface HDCP

Get whether HDCP is enabled on the input interface(s).

| Command Structure GET HDCP_S < IN1 ¦ IN2 ¦ ALL > | Parameters: ALL = Get the HDCP on/off status for all input interfaces |
| --- | --- |
| Response Structure HDCP_S < IN1 ¦ IN2 > < ON ¦ OFF > or HDCP_S IN1 < ON ¦ OFF > HDCP_S IN2 < ON ¦ OFF > |  |
| Command Example |  |
| GET HDCP_S IN1 |  |
| Response Example |  |
| HDCP_S IN1 OFF |  |
| Additional Notes: |  |

## 2.5 Get Input Signal HDCP Status

Retrieve the HDCP encryption status of the input signal(s).

| Command Structure GET VIDIN_HDCP < IN1 ¦ IN2 ¦ ALL > | Parameters: ALL = Retrieve the HDCP encryption status for all input signals |
| --- | --- |
| Response Structure VIDIN_HDCP < IN1 ¦ IN2 > < OFF ¦ HDCP1.4 ¦ HDCP2.2 > or VIDIN_HDCP IN1 < OFF ¦ HDCP1.4 ¦ HDCP2.2 > VIDIN_HDCP IN2 < OFF ¦ HDCP1.4 ¦ HDCP2.2 > |  |
| Command Example |  |
| GET VIDIN_HDCP IN1 |  |
| Response Example |  |
| VIDIN_HDCP IN1 OFF |  |
| Additional Notes: |  |

## 2.6 Set Output Interface HDCP

Set whether HDCP is enabled on an output interface.

| Command Structure SET HDCP < OUT1 ¦ OUT2 > < ON ¦ OFF > | Parameters: |
| --- | --- |
| Response Structure HDCP < OUT1 ¦ OUT2 > < ON ¦ OFF > |  |
| Command Example |  |
| SET HDCP OUT1 OFF |  |
| Response Example |  |
| HDCP OUT1 OFF |  |
| Additional Notes: Default: HDCP is enabled on all output interfaces. |  |

## 2.7 Get Output Interface HDCP

Retrieve whether HDCP is enabled on an output interface.

| Command Structure GET HDCP < OUT1 ¦ OUT2 ¦ ALL > | Parameters: ALL = Get the HDCP on/off status for all output interfaces |
| --- | --- |
| Response Structure HDCP < OUT1 ¦ OUT2 > < ON ¦ OFF > or HDCP OUT1 < ON ¦ OFF > HDCP OUT2 < ON ¦ OFF > |  |
| Command Example |  |
| GET HDCP OUT1 |  |
| Response Example |  |
| HDCP OUT1 OFF |  |
| Additional Notes: |  |

## 2.8 Get Output Signal HDCP Status

Get the HDCP encryption status of the output signal(s).

| Command Structure GET VIDOUT_HDCP < OUT1 ¦ OUT2 ¦ ALL > | Parameters: ALL = Get the HDCP encryption status for all output signals. |
| --- | --- |
| Response Structure VIDOUT_HDCP < OUT1 ¦ OUT2 > < OFF ¦ HDCP1.4 ¦ HDCP2.2 > or VIDOUT_HDCP OUT1 < OFF ¦ HDCP1.4 ¦ HDCP2.2 > VIDOUT_HDCP OUT2 < OFF ¦ HDCP1.4 ¦ HDCP2.2 > |  |
| Command Example |  |
| GET VIDOUT_HDCP OUT1 |  |
| Response Example |  |
| VIDOUT_HDCP OUT1 HDCP1.4 |  |
| Additional Notes: |  |

## 2.9 Get Input Connection Status

Retrieve the connection status of input interfaces.

| Command Structure GET VIDIN_CONNECT < IN1 ¦ IN2 ¦ ALL > | Parameters: ALL = Retrieve the connection status of all input interfaces. |
| --- | --- |
| Response Structure VIDIN_CONNECT < IN1 ¦ IN2 > < DISCONNECTED ¦ CONNECTED > or VIDIN_CONNECT IN1 < DISCONNECTED ¦ CONNECTED > VIDIN_CONNECT IN2 < DISCONNECTED ¦ CONNECTED > |  |
| Command Example |  |
| GET VIDIN_CONNECT IN2 |  |
| Response Example |  |
| VIDIN_CONNECT IN2 DISCONNECTED |  |
| Additional Notes: |  |

## 2.10 Get Input Signal Status

Get the status of input signals.

| Command Structure GET VIDIN_SIG < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 ¦ ALL > | Parameters: ALL = Get the status of all input signals |
| --- | --- |
| Response Structure VIDIN_SIG < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 > < NO ¦ VALID > or VIDIN_SIG IN1 < NO ¦ VALID > VIDIN_SIG IN2 < NO ¦ VALID > VIDIN_SIG BYOD1 < NO ¦ VALID > VIDIN_SIG BYOD2 < NO ¦ VALID > |  |
| Command Example |  |
| GET VIDIN_SIG IN2 |  |
| Response Example |  |
| VIDIN_SIG IN2 VALID |  |
| Additional Notes: |  |

## 2.11 Get Input Video Format

Retrieve the video format of the input signal.

| Command Structure GET VIDIN_FORMAT < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 ¦ ALL > | Parameters: ALL = Get the video formats of all input interfaces INVALID = No valid video signal detected on the input interface The format of the parameter VideoFormat: < WidthXHeight[P],FrameRate;HDRInfo;EncMode;ColorDepth > The value and meaning of each field are as follows: - Width = The width of the video (pixels) - Height = The height of the video (pixels) - P = Progressive scan format - FrameRate = Frame rate (frames per second) - HDRInfo = HDR information. The device does not support HDR, so this value is always NONE HDR. - EncMode = Encoding/color mode. Available values: RGB, YUV444, YUV422, H.264 - ColorDepth = Color depth. Currently only 8BIT is supported. - An actual example of VideoFormat: 1920X1080P,60;NONE HDR;RGB;8BIT |
| --- | --- |
| Response Structure VIDIN_FORMAT < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 > < INVALID ¦ VideoFormat > or VIDIN_FORMAT IN1 < INVALID ¦ VideoFormat > VIDIN_FORMAT IN2 < INVALID ¦ VideoFormat > VIDIN_FORMAT BYOD1 < INVALID ¦ VideoFormat > VIDIN_FORMAT BYOD2 < INVALID ¦ VideoFormat > |  |
| Command Example |  |
| GET VIDIN_FORMAT IN2 |  |
| Response Example |  |
| VIDIN_FORMAT IN2 3840X2160P,60;NONE HDR;RGB;8BIT |  |
| Additional Notes: |  |

## 2.12 Get Input Audio Format

Retrieve the audio format of the input signal.

| Command Structure GET AUDIN_FORMAT < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 ¦ ALL > | Parameters: ALL = Get the audio format of all input interfaces. INVALID = No valid audio signal detected on the input interface. The format of the parameter AudioFormat: < Encoding;SamplingRate;SamplingWidth > The value and meaning of every field are as follows: - Encoding = The audio encoding mode. Available values: PCM, AAC, MP3, OPUS - SamplingRate = Sampling rate, for example 48kHz - SamplingWidth = Sampling width. Available values: 8BIT, 16BIT Example of AudioFormat: PCM;48kHz;8BIT |
| --- | --- |
| Response Structure AUDIN_FORMAT < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 > < INVALID ¦ AudioFormat > or AUDIN_FORMAT IN1 < INVALID ¦ AudioFormat > AUDIN_FORMAT IN2 < INVALID ¦ AudioFormat > AUDIN_FORMAT BYOD1 < INVALID ¦ AudioFormat > AUDIN_FORMAT BYOD2 < INVALID ¦ AudioFormat > |  |
| Command Example |  |
| GET AUDIN_FORMAT IN2 |  |
| Response Example |  |
| AUDIN_FORMAT IN2 PCM;48kHz;8BIT |  |
| Additional Notes: |  |

## 2.13 Set Output Volume

Set the volume of all output audio signals at once.

| Command Structure SET VOLGAIN_DATA < OutputVolume > | Parameters: OutputVolume is in dB. Valid range: -100 to 6. |
| --- | --- |
| Response Structure VOLGAIN_DATA < OutputVolume > |  |
| Command Example |  |
| SET VOLGAIN_DATA -3 |  |
| Response Example |  |
| VOLGAIN_DATA -3 |  |
| Additional Notes: Default: 0 |  |

## 2.14 Get Output Volume

Get the volume level of the output audio signal

| Command Structure GET VOLGAIN_DATA | Parameters: |
| --- | --- |
| Response Structure VOLGAIN_DATA < OutputVolume > |  |
| Command Example |  |
| GET VOLGAIN_DATA |  |
| Response Example |  |
| VOLGAIN_DATA -3 |  |
| Additional Notes: |  |

## 2.15 Set Output Mute

Mute or unmute all output audio signals simultaneously.

| Command Structure SET AUDIO_MUTE < ON ¦ OFF > | Parameters: ON = Enable output mute |
| --- | --- |
| Response Structure AUDIO_MUTE < ON ¦ OFF > |  |
| Command Example |  |
| SET AUDIO_MUTE ON |  |
| Response Example |  |
| AUDIO_MUTE ON |  |
| Additional Notes: Default: OFF (not muted) |  |

## 2.16 Get Output Mute

Get the mute setting for the output audio signal.

| Command Structure GET AUDIO_MUTE | Parameters: |
| --- | --- |
| Response Structure AUDIO_MUTE < ON ¦ OFF > |  |
| Command Example |  |
| GET AUDIO_MUTE |  |
| Response Example |  |
| AUDIO_MUTE ON |  |
| Additional Notes: |  |

## 2.17 Get Output Connection Status

Get the connection status of the output ports (whether a display is connected).

| Command Structure GET VIDOUT_CONNECT < OUT1 ¦ OUT2 ¦ ALL > | Parameters: ALL = Get the connection status of both output ports. |
| --- | --- |
| Response Structure VIDOUT_CONNECT < OUT1 ¦ OUT2 > < CONNECTED ¦ DISCONNECTED > 或 VIDOUT_CONNECT OUT1 < CONNECTED ¦ DISCONNECTED > VIDOUT_CONNECT OUT2 < CONNECTED ¦ DISCONNECTED > |  |
| Command Example |  |
| GET VIDOUT_CONNECT OUT2 |  |
| Response Example |  |
| VIDOUT_CONNECT OUT2 CONNECTED |  |
| Additional Notes: |  |

## 2.18 Set MST

Configure the MST feature of the USB-C input. MST is actually enabled only when both of the following conditions are met:
1. This setting is set to MST
2. Both HDMI outputs are connected
When MST is enabled, the device's video switching logic will behave differently than normal.

| Command Structure SET USBC_DM < SST ¦ MST > | Parameters: SST = USB-C input operates in SST mode; MST is not supported MST = USB-C input operates in MST mode |
| --- | --- |
| Response Structure USBC_DM < SST ¦ MST > |  |
| Command Example |  |
| SET USBC_DM MST |  |
| Response Example |  |
| USBC_DM MST |  |
| Additional Notes: Default value: SST (MST disabled) |  |

## 2.19 Get MST

Retrieve the MST (Multi-Stream Transport) capability of the USB-C input port.

| Command Structure GET USBC_DM | Parameters: |
| --- | --- |
| Response Structure USBC_DM < SST ¦ MST > |  |
| Command Example |  |
| GET USBC_DM |  |
| Response Example |  |
| USBC_DM MST |  |
| Additional Notes: |  |

# 3. Video Switching

## 3.1 Set Automatic Switching

Enable or disable the automatic switching feature.
When automatic switching is enabled, the device supports the following functions:
1. Automatic switching of video sources
2. Automatic switching of multi-view layouts
3. Fallback display of video sources

| Command Structure SET AUTOSW_FN < ON ¦ OFF > | Parameters: ON = Automatic switching enabled |
| --- | --- |
| Response Structure AUTOSW_FN < ON ¦ OFF > |  |
| Command Example |  |
| SET AUTOSW_FN OFF |  |
| Response Example |  |
| AUTOSW_FN OFF |  |
| Additional Notes: Default: ON |  |

## 3.2 Get Automatic Switching Status

Retrieve whether the automatic switching feature is enabled (ON) or disabled (OFF).

| Command Structure GET AUTOSW_FN | Parameters: |
| --- | --- |
| Response Structure AUTOSW_FN < ON ¦ OFF > |  |
| Command Example |  |
| GET AUTOSW_FN |  |
| Response Example |  |
| AUTOSW_FN OFF |  |
| Additional Notes: |  |

## 3.3 Set Multi-View Function

Turn the multi-view function on or off

| Command Structure SET MV_ENABLE < ON ¦ OFF > | Parameters: ON = Enable the multi-view function |
| --- | --- |
| Response Structure MV_ENABLE < ON ¦ OFF > |  |
| Command Example |  |
| SET MV_ENABLE OFF |  |
| Response Example |  |
| MV_ENABLE OFF |  |
| Additional Notes: Default: ON |  |

## 3.4 Get Multi-View Function Status

Get the on/off status of the multi-view function

| Command Structure GET MV_ENABLE | Parameters: |
| --- | --- |
| Response Structure MV_ENABLE < ON ¦ OFF > |  |
| Command Example |  |
| GET MV_ENABLE |  |
| Response Example |  |
| MV_ENABLE OFF |  |
| Additional Notes: |  |

## 3.5 Switch Input to All Outputs

Switch an input to display full-screen on all outputs

| Command Structure SET SW < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 > | Parameters: |
| --- | --- |
| Response Structure SW < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 > OUT1 SW < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 > OUT2 |  |
| Command Example |  |
| SET SW IN1 |  |
| Response Example |  |
| SW IN1 OUT1 SW IN1 OUT2 |  |
| Additional Notes: |  |

## 3.6 Switch Input to an Output

Switch an input to full-screen display on a specified output.
If the specified output is not connected, the device will display the input full-screen on the currently connected output. The response message always returns information about the output that is actually displaying the source.

| Command Structure SET SW < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 > < OUT1 ¦ OUT2 > | Parameters: |
| --- | --- |
| Response Structure SW < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 > < OUT1 ¦ OUT2 > |  |
| Command Example |  |
| SET SW IN1 OUT1 |  |
| Response Example |  |
| SW IN1 OUT1 |  |
| Additional Notes: If only one output is currently connected, the second parameter is ignored and the specified input will be displayed on the single connected output. |  |

## 3.7 Display GUIDE Screen

Show the GUIDE screen on all outputs.
This means no video source is being shown. In this state, if auto-standby is enabled, the device will enter standby automatically after the GUIDE screen has been shown continuously for the auto-standby timeout period.
Note: if the auto-standby timeout is set to 0, this command will cause the device to enter standby immediately.

| Command Structure SET SW GUIDE | Parameters: |
| --- | --- |
| Response Structure SW GUIDE |  |
| Command Example |  |
| SET SW GUIDE |  |
| Response Example |  |
| SW GUIDE |  |
| Additional Notes: |  |

## 3.8 Get Display Status

Retrieve the display status of an output.

| Command Structure GET SW < OUT1 ¦ OUT2 ¦ ALL > | Parameters: ALL = Retrieve display status for all outputs GUIDE = The output is not showing a video source and is displaying the Guide screen MV = The output is in multi-view (multi-picture) mode DISCONNECTED = The output is not connected |
| --- | --- |
| Response Structure SW < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 ¦ GUIDE ¦ MV ¦ DISCONNECTED > < OUT1 ¦ OUT2 > 或 SW < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 ¦ GUIDE ¦ MV ¦ DISCONNECTED > OUT1 SW < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 ¦ GUIDE ¦ MV ¦ DISCONNECTED > OUT2 |  |
| Command Example |  |
| GET SW OUT1 |  |
| Response Example |  |
| SW IN1 OUT1 |  |
| Additional Notes: |  |

## 3.9 Set Current Layout

Switch the multi-view layout. The parameter is the layout name.

| Command Structure SET MV_LAYOUT < FULLSCREEN ¦ DUALVIEW > | Parameters: |
| --- | --- |
| Response Structure MV_LAYOUT < FULLSCREEN ¦ DUALVIEW > |  |
| Command Example |  |
| SET MV_LAYOUT DUALVIEW |  |
| Response Example |  |
| MV_LAYOUT DUALVIEW |  |
| Additional Notes: |  |

## 3.10 Set Current Layout With ID

Switch the multiview layout.
The parameter is the layout ID in hexadecimal format.

| Command Structure SET MV_LAYOUT < 0X100 ¦ 0X101 > | Parameters: The four hexadecimal IDs correspond to the four layout names defined in the previous API. |
| --- | --- |
| Response Structure MV_LAYOUT < 0X100 ¦ 0X101 > |  |
| Command Example |  |
| SET MV_LAYOUT 0X101 |  |
| Response Example |  |
| MV_LAYOUT 0X101 |  |
| Additional Notes: |  |

## 3.11 Get Current Layout ID and Name

Get the name of the current multi-view layout.

| Command Structure GET MV_LAYOUT | Parameters: |
| --- | --- |
| Response Structure MV_LAYOUT < < 0X100 FULLSCREEN > ¦ < 0X101 DUALVIEW > > |  |
| Command Example |  |
| GET MV_LAYOUT |  |
| Response Example |  |
| MV_LAYOUT 0X101 DUALVIEW |  |
| Additional Notes: |  |

## 3.12 Set Layout and Video Sources

Switch multi-view layouts and set the video source for each sub-window.

| Command Structure SET MV_WIN_SRC < FULLSCREEN ¦ DUALVIEW ¦ CURRENT > < WinNo1 > < Src1 > < WinNo2 > < Src2 > ... | Parameters: CURRENT = Keep the current layout unchanged; only change the video source(s) for the specified window(s). WinNo = Win1 Win2 Src1 Src2... possible values: < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 ¦ NONE > - NONE = Do not display any video source in that sub-window; the sub-window remains empty |
| --- | --- |
| Response Structure MV_WIN_SRC < FULLSCREEN ¦ DUALVIEW > < WinNo1 > < Src1 > < WinNo2 > < Src2 > ... |  |
| Command Example |  |
| SET MV_WIN_SRC DUALVIEW WIN1 IN1 WIN2 IN2 |  |
| Response Example |  |
| MV_WIN_SRC DUALVIEW WIN1 IN1 WIN2 IN2 |  |
| Additional Notes: |  |

## 3.13 Get Layout and Video Sources

Get the current layout and the video source for each sub-window.

| Command Structure GET MV_WIN_SRC | Parameters: NONE = Currently connected to dual outputs; multi-view is not supported. |
| --- | --- |
| Response Structure MV_WIN_SRC NONE 或 MV_WIN_SRC < FULLSCREEN ¦ DUALVIEW > < WinNo1 > < Src1 > < WinNo2 > < Src2 > ... |  |
| Command Example |  |
| GET MV_WIN_SRC |  |
| Response Example |  |
| MV_WIN_SRC DUALVIEW WIN1 IN1 WIN2 IN2 |  |
| Additional Notes: When connected to a single output but multi-view is disabled, this API returns normally. |  |

## 3.14 Get Video Source of a Specified Window

Get the video source for a specified sub-window.

| Command Structure GET MV_WIN_SRC CURRENT WinNo | Parameters: NONE = Currently connected to dual outputs; multi-view is not supported. |
| --- | --- |
| Response Structure MV_WIN_SRC NONE 或 MV_WIN_SRC CURRENT < WinNo > < Src > |  |
| Command Example |  |
| GET MV_WIN_SRC CURRENT WIN2 |  |
| Response Example |  |
| MV_WIN_SRC CURRENT WIN2 IN2 |  |
| Additional Notes: When connected to a single output but multi-view is disabled, this API returns normally. This API is a subset of the previous API, used to support SYN-TOUCH10. |  |

# 4. Audio and Other Signal Switching

## 4.1 Set Audio Auto-Switching

Enable or disable audio auto-switching.

| Command Structure SET AUDIO_AUTOSW_FN < ON ¦ OFF > | Parameters: Default value: ON |
| --- | --- |
| Response Structure AUDIO_AUTOSW_FN < ON ¦ OFF > |  |
| Command Example |  |
| SET AUDIO_AUTOSW_FN OFF |  |
| Response Example |  |
| AUDIO_AUTOSW_FN OFF |  |
| Additional Notes: |  |

## 4.2 Get Audio Auto-Switch Status

| Command Structure GET AUDIO_AUTOSW_FN | Parameters: |
| --- | --- |
| Response Structure AUDIO_AUTOSW_FN < ON ¦ OFF > |  |
| Command Example |  |
| GET AUDIO_AUTOSW_FN |  |
| Response Example |  |
| AUDIO_AUTOSW_FN OFF |  |
| Additional Notes: |  |

## 4.3 Switch to Wired Audio Source

When audio auto-switching is disabled, use this API to force the device to always output audio from a wired source.

| Command Structure SET AUDIOSW < IN1 ¦ IN2 > | Parameters: STOP = Currently in the Guide screen state; the device is not actually outputting audio. |
| --- | --- |
| Response Structure AUDIOSW < IN1 ¦ IN2 > [ STOP ] |  |
| Command Example |  |
| SET AUDIOSW IN2 |  |
| Response Example |  |
| AUDIOSW IN2 |  |
| Additional Notes: |  |

## 4.4 Switch to BYOD Audio Source

When automatic audio switching is off, make the device temporarily output audio from a BYOD source. The device will play the BYOD source’s audio until that BYOD source disconnects, then it will switch back to the previously set wired audio source. In short, this is a temporary switch.

| Command Structure SET AUDIOSW < BYOD1 ¦ BYOD2 > | Parameters: |
| --- | --- |
| Response Structure AUDIOSW < BYOD1 ¦ BYOD2 > |  |
| Command Example |  |
| SET AUDIOSW BYOD1 |  |
| Response Example |  |
| AUDIOSW BYOD1 |  |
| Additional Notes: |  |

## 4.5 Get Current Audio Source

Retrieve the audio source that the device is currently outputting.
If the device is showing the Guide screen and audio output is stopped, this API returns the most recent wired audio source that was output.

| Command Structure GET AUDIOSW | Parameters: STOP = Device is showing the Guide screen and is not actually outputting audio |
| --- | --- |
| Response Structure AUDIOSW < IN1 ¦ IN2 ¦ BYOD1 ¦ BYOD2 > [ STOP ] |  |
| Command Example |  |
| GET AUDIOSW |  |
| Response Example |  |
| AUDIOSW IN1 |  |
| Additional Notes: |  |

## 4.6 Set USB Auto-Switch

Configure the device to automatically switch the USB host.

| Command Structure SET USB_AUTOSW_FN < ON ¦ OFF > | Parameters: ON = Enable USB automatic switching |
| --- | --- |
| Response Structure USB_AUTOSW_FN < ON ¦ OFF > |  |
| Command Example |  |
| SET USB_AUTOSW_FN ON |  |
| Response Example |  |
| USB_AUTOSW_FN ON |  |
| Additional Notes: Default: ON |  |

## 4.7 Get USB Automatic Switch Status

Retrieve the ON/OFF status of the USB automatic switching function.

| Command Structure GET USB_AUTOSW_FN | Parameters: |
| --- | --- |
| Response Structure USB_AUTOSW_FN < ON ¦ OFF > |  |
| Command Example |  |
| GET USB_AUTOSW_FN |  |
| Response Example |  |
| USB_AUTOSW_FN ON |  |
| Additional Notes: |  |

## 4.8 Set USB Host

Manually switch the USB Host

| Command Structure SET USB_HOST < USBCIN ¦ USBHOST ¦ WIRELESS > | Parameters: WIRELESS = USB Host connected to the main chipset for Wireless Conference |
| --- | --- |
| Response Structure USB_HOST < USBCIN ¦ USBHOST ¦ WIRELESS > |  |
| Command Example |  |
| SET USB_HOST USBHOST1 |  |
| Response Example |  |
| USB_HOST USBHOST1 |  |
| Additional Notes: |  |

## 4.9 Get USB Host

Retrieve the current USB HOST.

| Command Structure GET USB_HOST | Parameters: |
| --- | --- |
| Response Structure USB_HOST < USBCIN ¦ USBHOST ¦ WIRELESS > |  |
| Command Example |  |
| GET USB_HOST |  |
| Response Example |  |
| USB_HOST USBHOST1 |  |
| Additional Notes: |  |

# 5. Sleep/Standby and Peripheral Control

## 5.1 Set Automatic Standby

Configure the device's automatic standby feature.
When automatic standby is enabled, if the device continuously displays the Guide screen for a certain period (determined by the standby timeout value), it will send a power-off command to all connected displays and disable HDMI +5V and TMDS signals.

| Command Structure SET AUTO_STANDBY_FN < ON ¦ OFF > | Parameters: ON = Enable automatic standby |
| --- | --- |
| Response Structure AUTO_STANDBY_FN < ON ¦ OFF > |  |
| Command Example |  |
| SET AUTO_STANDBY_FN OFF |  |
| Response Example |  |
| AUTO_STANDBY_FN OFF |  |
| Additional Notes: Default: ON |  |

## 5.2 Get Automatic Standby

Retrieve the device's automatic standby switch status.

| Command Structure GET AUTO_STANDBY_FN | Parameters: |
| --- | --- |
| Response Structure AUTO_STANDBY_FN < ON ¦ OFF > |  |
| Command Example |  |
| GET AUTO_STANDBY_FN |  |
| Response Example |  |
| AUTO_STANDBY_FN OFF |  |
| Additional Notes: |  |

## 5.3 Set Automatic Standby Timeout

Set the automatic standby timeout value.

| Command Structure SET AUTO_STANDBY_D < TimeOut > | Parameters: TimeOut — valid range: 0-3600 (seconds) |
| --- | --- |
| Response Structure AUTO_STANDBY_D < TimeOut > |  |
| Command Example |  |
| SET AUTO_STANDBY_D |  |
| Response Example |  |
| AUTO_STANDBY_D 100 |  |
| Additional Notes: Default value: 120 (seconds) |  |

## 5.4 Get Auto-Standby Timeout

Retrieve the auto-standby timeout value.

| Command Structure GET AUTO_STANDBY_D | Parameters: |
| --- | --- |
| Response Structure AUTO_STANDBY_D < TimeOut > |  |
| Command Example |  |
| GET AUTO_STANDBY_D |  |
| Response Example |  |
| AUTO_STANDBY_D 100 |  |
| Additional Notes: |  |

## 5.5 Set Device Power On or Standby

Manually control the device to power on or enter standby.

| Command Structure SET STANDBY < ON ¦ OFF > | Parameters: ON = Power on OFF = Standby |
| --- | --- |
| Response Structure STANDBY < ON ¦ OFF > |  |
| Command Example |  |
| SET STANDBY ON |  |
| Response Example |  |
| STANDBY ON |  |
| Additional Notes: |  |

## 5.6 Get Device Standby

Retrieve whether the device is powered on.

| Command Structure GET STANDBY | Parameters: |
| --- | --- |
| Response Structure STANDBY < ON ¦ OFF > |  |
| Command Example |  |
| GET STANDBY |  |
| Response Example |  |
| STANDBY ON |  |
| Additional Notes: |  |

## 5.7 Send CEC Power On/Off Command

Send a CEC power-on or power-off command to the specified output interface.

| Command Structure SET CEC_PWR < OUT1 ¦ OUT2 > < ON ¦ OFF > | Parameters: |
| --- | --- |
| Response Structure CEC_PWR < OUT1 ¦ OUT2 > < ON ¦ OFF > |  |
| Command Example |  |
| SET CEC_PWR OUT1 ON |  |
| Response Example |  |
| CEC_PWR OUT1 ON |  |
| Additional Notes: |  |

## 5.8 Set CEC Power On/Off Command

Set the CEC power-on or power-off command for the specified output interface.

| Command Structure SET CECCMD_EDIT < OUT1 ¦ OUT2 > < ON ¦ OFF > < CECCode > | Parameters: CECCode is the CEC message as a hexadecimal string with no spaces between adjacent bytes. |
| --- | --- |
| Response Structure CECCMD_EDIT < OUT1 ¦ OUT2 > < ON ¦ OFF > < CECCode > |  |
| Command Example |  |
| SET CECCMD_EDIT OUT1 ON 4004 |  |
| Response Example |  |
| CECCMD_EDIT OUT1 ON 4004 |  |
| Additional Notes: Default values: Power on: 4004 Power off: FF36 |  |

## 5.9 Get CEC Power On/Off Command

Retrieve the CEC power-on or power-off command for the specified output interface.

| Command Structure GET CECCMD_EDIT < OUT1 ¦ OUT2 > < ON ¦ OFF > | Parameters: |
| --- | --- |
| Response Structure CECCMD_EDIT < OUT1 ¦ OUT2 > < ON ¦ OFF > < CECCode > |  |
| Command Example |  |
| GET CECCMD_EDIT OUT1 ON |  |
| Response Example |  |
| CECCMD_EDIT OUT1 ON 4004 |  |
| Additional Notes: |  |

## 5.10 Get CEC Commands

Retrieve the CEC power-on and power-off commands for the specified output interface.

| Command Structure GET CECCMD_EDIT < OUT1 ¦ OUT2 > | Parameters: |
| --- | --- |
| Response Structure CECCMD_EDIT < OUT1 ¦ OUT2 > ON < CECCode > CECCMD_EDIT < OUT1 ¦ OUT2 > OFF < CECCode > |  |
| Command Example |  |
| GET CECCMD_EDIT OUT1 |  |
| Response Example |  |
| CECCMD_EDIT OUT1 ON 4004 CECCMD_EDIT OUT1 OFF FF36 |  |
| Additional Notes: |  |

## 5.11 Get All CEC Commands

Get the CEC power-on and power-off commands for all output interfaces.

| Command Structure GET CECCMD_EDIT ALL | Parameters: |
| --- | --- |
| Response Structure CECCMD_EDIT OUT1 ON < CECCode > CECCMD_EDIT OUT1 OFF < CECCode > CECCMD_EDIT OUT2 ON < CECCode > CECCMD_EDIT OUT2 OFF < CECCode > |  |
| Command Example |  |
| GET CECCMD_EDIT ALL |  |
| Response Example |  |
| CECCMD_EDIT OUT1 ON 4004 CECCMD_EDIT OUT1 OFF FF36 CECCMD_EDIT OUT2 ON 4004 CECCMD_EDIT OUT2 OFF FF36 |  |
| Additional Notes: |  |

## 5.12 Send CEC Command String

Sends a CEC command string directly to a specified output interface.

| Command Structure SET CEC_CMD < OUT1 ¦ OUT2 > < CECCode > | Parameters: |
| --- | --- |
| Response Structure CEC_CMD < OUT1 ¦ OUT2 > < CECCode > |  |
| Command Example |  |
| SET CEC_CMD OUT1 4004 |  |
| Response Example |  |
| CEC_CMD OUT1 4004 |  |
| Additional Notes: |  |

## 5.13 Set UART Parameters

Set the four UART parameters: baud rate, parity, data bits, and stop bits.

| Command Structure SET UART_CFG < BaudRate > < Parity > < DataBit > < StopBit > | Parameters: BaudRate is the baud rate; valid values: 9600, 19200, 38400, 57600, 115200 Parity indicates parity checking; valid values: NONE, ODD, EVEN DataBit is the number of data bits; valid values: 7, 8 StopBit is the number of stop bits; valid values: 1, 2 |
| --- | --- |
| Response Structure UART_CFG < BaudRate > < Parity > < DataBit > < StopBit > |  |
| Command Example |  |
| SET UART_CFG 115200 NONE 8 1 |  |
| Response Example |  |
| UART_CFG 115200 NONE 8 1 |  |
| Additional Notes: Default: 115200 NONE 8 1 |  |

## 5.14 Get UART Parameters

Get the UART's baud rate and other serial port parameters.

| Command Structure GET UART_CFG | Parameters: |
| --- | --- |
| Response Structure UART_CFG < BaudRate > < Parity > < DataBit > < StopBit > |  |
| Command Example |  |
| GET UART_CFG |  |
| Response Example |  |
| UART_CFG 115200 NONE 8 1 |  |
| Additional Notes: |  |

## 5.15 Set UART Baud Rate

Sets the UART baud rate parameter.

| Command Structure SET UART_B < BaudRate > | Parameters: |
| --- | --- |
| Response Structure UART_B < BaudRate > |  |
| Command Example |  |
| SET UART_B 115200 |  |
| Response Example |  |
| UART_B 115200 |  |
| Additional Notes: |  |

## 5.16 Get UART Baud Rate

Retrieve the UART (serial port) baud rate.

| Command Structure GET UART_B | Parameters: |
| --- | --- |
| Response Structure UART_B < BaudRate > |  |
| Command Example |  |
| GET UART_B |  |
| Response Example |  |
| UART_B 115200 |  |
| Additional Notes: |  |

## 5.17 Set UART Command

Set a UART (serial) command

| Command Structure SET UART_CMD < CmdName > < HEX ¦ STR > < CmdStr > | Parameters: CmdName = command name. In addition to ON and OFF, this can be other custom names such as VolumeUp. HEX = command in hexadecimal format STR = command in string format CmdStr = the command string. For hexadecimal command strings, adjacent bytes do not need to be separated by spaces. |
| --- | --- |
| Response Structure UART_CMD < CmdName > < HEX ¦ STR > < CmdStr > |  |
| Command Example |  |
| SET UART_CMD ON STR POWERON |  |
| Response Example |  |
| UART_CMD ON STR POWERON |  |
| Additional Notes: By default, no UART (serial) commands are defined. |  |

## 5.18 Delete UART Command

Delete a UART command

| Command Structure SET UART_CMD_D < CmdName > | Parameters: |
| --- | --- |
| Response Structure UART_CMD_D < CmdName > |  |
| Command Example |  |
| SET UART_CMD_D ON |  |
| Response Example |  |
| UART_CMD_D ON |  |
| Additional Notes: |  |

## 5.19 Get UART Commands

Retrieve one or all UART (serial) commands.

| Command Structure GET UART_CMD < CmdName ¦ ALL > | Parameters: |
| --- | --- |
| Response Structure UART_CMD < CmdName > < HEX ¦ STR > < CmdStr > 或 UART_CMD < CmdName1 > < HEX ¦ STR > < CmdStr1 > UART_CMD < CmdName2 > < HEX ¦ STR > < CmdStr2 > ... |  |
| Command Example |  |
| GET UART_CMD ON |  |
| Response Example |  |
| UART_CMD ON STR POWERON |  |
| Additional Notes: |  |

## 5.20 Send UART Command

Send a previously defined UART (serial) command.

| Command Structure SET UART_CMD_S < CmdName > | Parameters: |
| --- | --- |
| Response Structure UART_CMD_S < CmdName > |  |
| Command Example |  |
| SET UART_CMD_S ON |  |
| Response Example |  |
| UART_CMD_S ON |  |
| Additional Notes: |  |

## 5.21 Send UART Command String

Directly send UART data.

| Command Structure SET UART_S < HEX ¦ STR > < CmdStr > | Parameters: |
| --- | --- |
| Response Structure UART_S < HEX ¦ STR > < CmdStr > |  |
| Command Example |  |
| SET UART_S STR POWERON |  |
| Response Example |  |
| UART_S STR POWERON |  |
| Additional Notes: |  |

## 5.22 Set UART Power On/Off Function

Configure whether the UART participates in sending power on/off commands.
When the UART is operating in external control mode, if this setting is enabled, then when the device automatically or manually enters or exits standby, it will, in addition to sending CEC commands, also send the corresponding commands via the UART.

| Command Structure SET UARTPWR_FN < ON ¦ OFF > | Parameters: |
| --- | --- |
| Response Structure UARTPWR_FN < ON ¦ OFF > |  |
| Command Example |  |
| SET UARTPWR_FN ON |  |
| Response Example |  |
| UARTPWR_FN ON |  |
| Additional Notes: Default: OFF |  |

## 5.23 Get UART Power On/Off Function

Get whether the UART participates in sending power on/off commands.

| Command Structure GET UARTPWR_FN | Parameters: |
| --- | --- |
| Response Structure UARTPWR_FN < ON ¦ OFF > |  |
| Command Example |  |
| GET UARTPWR_FN |  |
| Response Example |  |
| UARTPWR_FN ON |  |
| Additional Notes: |  |

## 5.24 Send CEC and/or RS232 Commands

Send CEC and/or RS232 commands simultaneously.

| Command Structure SET SEND_CMD < ON ¦ OFF > < RS232 ¦ CEC ¦ ALL > | Parameters: RS232 = Send only RS232 commands CEC = Send only CEC commands ALL = Send both RS232 and CEC commands simultaneously |
| --- | --- |
| Response Structure SEND_CMD < ON ¦ OFF > < RS232 ¦ CEC ¦ ALL > |  |
| Command Example |  |
| SET SEND_CMD ON CEC |  |
| Response Example |  |
| SEND_CMD ON CEC |  |
| Additional Notes: |  |

## 5.25 Set UART Operating Mode

The UART can be used for external control or to provide API services (i.e., to receive control). This API configures that mode.

| Command Structure SET UART_MODE < API ¦ COM > | Parameters: API = The UART is used to provide API services externally; in this mode, functions in earlier APIs that involve sending actual UART data will be disabled. COM = The UART is used for external control |
| --- | --- |
| Response Structure UART_MODE < API ¦ COM > |  |
| Command Example |  |
| SET UART_MODE API |  |
| Response Example |  |
| UART_MODE API |  |
| Additional Notes: Default: COM |  |

## 5.26 Get UART Operating Mode

Retrieve the UART (serial port) operating mode.

| Command Structure GET UART_MODE | Parameters: |
| --- | --- |
| Response Structure UART_MODE < API ¦ COM > |  |
| Command Example |  |
| GET UART_MODE |  |
| Response Example |  |
| UART_MODE API |  |
| Additional Notes: |  |

## 5.27 Get MUTE Input

Get the status of the MUTE input.

| Command Structure GET MUTE_CONTROL | Parameters: HIGH: The MUTE input is shorted, forcing mute to be active. |
| --- | --- |
| Response Structure MUTE_CONTROL < HIGH ¦ LOW > |  |
| Command Example |  |
| GET MUTE_CONTROL |  |
| Response Example |  |
| MUTE_CONTROL LOW |  |
| Additional Notes: |  |

# 6. Network and BYOD Related Settings

## 6.1 Set Network Isolation

When network isolation mode is enabled, the device's network ports are separated into two independent VLANs, which can be used to connect to different networks.
This API is used to set whether network isolation is enabled.

| Command Structure SET NET_ISOLATION < ON ¦ OFF > | Parameters: ON = Enable network isolation |
| --- | --- |
| Response Structure NET_ISOLATION < ON ¦ OFF > |  |
| Command Example |  |
| SET NET_ISOLATION ON |  |
| Response Example |  |
| NET_ISOLATION ON |  |
| Additional Notes: Default: OFF |  |

## 6.2 Get Network Isolation Settings

Retrieve whether network isolation mode is enabled.

| Command Structure GET NET_ISOLATION | Parameters: |
| --- | --- |
| Response Structure NET_ISOLATION < ON ¦ OFF > |  |
| Command Example |  |
| GET NET_ISOLATION |  |
| Response Example |  |
| NET_ISOLATION ON |  |
| Additional Notes: |  |

## 6.3 Set IP Address - DHCP

Set the DHCP IP address

| Command Structure SET IPADDR < CONTROL ¦ SERVICE > DHCP | Parameters: CONTROL = Sets the IP address for the primary/control network SERVICE = Sets the IP address for the service network when in network isolation mode |
| --- | --- |
| Response Structure IPADDR < CONTROL ¦ SERVICE > DHCP |  |
| Command Example |  |
| SET IPADDR CONTROL DHCP |  |
| Response Example |  |
| IPADDR CONTROL DHCP |  |
| Additional Notes: By default, both networks use DHCP mode |  |

## 6.4 Set IP Address - Static

Set a static IP address.

| Command Structure SET IPADDR < CONTROL ¦ SERVICE > STATIC < IPAddress > < NetMask > [ Gateway [ DNSServer ] ] | Parameters: Gateway and DNS server are optional parameters. |
| --- | --- |
| Response Structure IPADDR < CONTROL ¦ SERVICE > STATIC < IPAddress > < NetMask > [ Gateway [ DNSServer ] ] |  |
| Command Example |  |
| SET IPADDR SERVICE STATIC 192.168.1.34 255.255.255.0 192.168.1.1 192.168.1.2 |  |
| Response Example |  |
| IPADDR SERVICE STATIC 192.168.1.34 255.255.255.0 192.168.1.1 192.168.1.2 |  |
| Additional Notes: |  |

## 6.5 Get IP Address

Get the device's IP address.

| Command Structure GET IPADDR < CONTROL ¦ SERVICE ¦ ALL > | Parameters: ALL = Get the IP addresses for both network interfaces |
| --- | --- |
| Response Structure IPADDR < CONTROL ¦ SERVICE > < DHCP ¦ STATIC > < IPAddress > < NetMask > [ Gateway [ DNSServer ] ] 或 IPADDR CONTROL < DHCP ¦ STATIC > < IPAddress > < NetMask > [ Gateway [ DNSServer ] ] IPADDR SERVICE < DHCP ¦ STATIC > < IPAddress > < NetMask > [ Gateway [ DNSServer ] ] |  |
| Command Example |  |
| GET IPADDR SERVICE |  |
| Response Example |  |
| IPADDR SERVICE STATIC 192.168.1.34 255.255.255.0 192.168.1.1 192.168.1.2 |  |
| Additional Notes: |  |

## 6.6 Configure USB-C Network Adapter Function

This API enables or disables the network adapter on the USB-C port.

| Command Structure SET USBNIC < IN1 ¦ IN2 > < ON ¦ OFF > | Parameters: ON = Enable the USB port network adapter |
| --- | --- |
| Response Structure USBNIC < IN1 ¦ IN2 > < ON ¦ OFF > |  |
| Command Example |  |
| SET USBNIC IN1 OFF |  |
| Response Example |  |
| USBNIC IN1 OFF |  |
| Additional Notes: Default: ON |  |

## 6.7 Get USB Network Adapter Switch Status

Get the on/off status of the USB-C interface network adapter.

| Command Structure GET USBNIC < IN1 ¦ IN2 ¦ ALL > | Parameters: ALL = Get the on/off status of both USB network interfaces IN1 = USB-C IN port IN2 = USB HOST port |
| --- | --- |
| Response Structure USBNIC < IN1 ¦ IN2 > < ON ¦ OFF > 或 USBNIC IN1 < ON ¦ OFF > USBNIC IN2 < ON ¦ OFF > |  |
| Command Example |  |
| GET USBNIC IN1 |  |
| Response Example |  |
| USBNIC IN1 OFF |  |
| Additional Notes: |  |

## 6.8 Configure HTTPS

| Command Structure SET HTTPS < ON ¦ OFF > | Parameters: ON = Enables HTTPS |
| --- | --- |
| Response Structure HTTPS < ON ¦ OFF > |  |
| Command Example |  |
| SET HTTPS ON |  |
| Response Example |  |
| HTTPS ON |  |
| Additional Notes: Default: ON |  |

## 6.9 Get HTTPS Switch Status

| Command Structure GET HTTPS | Parameters: |
| --- | --- |
| Response Structure HTTPS < ON ¦ OFF > |  |
| Command Example |  |
| GET HTTPS |  |
| Response Example |  |
| HTTPS ON |  |
| Additional Notes: |  |

## 6.10 Set Telnet over TLS

| Command Structure SET TELNETS < ON ¦ OFF > | Parameters: ON = Enable Telnet over TLS |
| --- | --- |
| Response Structure TELNETS < ON ¦ OFF > |  |
| Command Example |  |
| SET TELNETS ON |  |
| Response Example |  |
| TELNETS ON |  |
| Additional Notes: Default: OFF |  |

## 6.11 Get Telnet over TLS Status

| Command Structure GET TELNETS | Parameters: |
| --- | --- |
| Response Structure TELNETS < ON ¦ OFF > |  |
| Command Example |  |
| GET TELNETS |  |
| Response Example |  |
| TELNETS ON |  |
| Additional Notes: |  |

## 6.12 Set Wi-Fi Operating Mode

Set the Wi‑Fi module's frequency band and channel

| Command Structure SET WIFI_MODE < WiFiBand > < WifiChannel ¦ AUTO > | Parameters: WiFiBand = 2 5 — corresponds to the 2.4 GHz and 5 GHz bands, respectively. WiFiChannel is the Wi‑Fi channel; its valid values depend on the band: 2.4G band: 1 2 3 4 5 6 7 8 9 10 11 5G band: 36 40 44 48 149 153 157 161 AUTO = the device automatically selects an appropriate channel |
| --- | --- |
| Response Structure WIFI_MODE < WiFiBand > < WifiChannel > [ AUTO ] |  |
| Command Example |  |
| SET WIFI_MODE 5 AUTO |  |
| Response Example |  |
| WIFI_MODE 5 161 AUTO |  |
| Additional Notes: Default: 5 AUTO That is, 5 GHz band with automatic channel selection. |  |

## 6.13 Get Wi‑Fi Operating Mode

| Command Structure GET WIFI_MODE | Parameters: |
| --- | --- |
| Response Structure WIFI_MODE < WiFiBand > < WifiChannel > [ AUTO ] |  |
| Command Example |  |
| GET WIFI_MODE |  |
| Response Example |  |
| WIFI_MODE 5 161 AUTO |  |
| Additional Notes: |  |

## 6.14 Set Wi-Fi Password

Set the password for the built-in soft AP used for wireless screen casting.

| Command Structure SET SOFTAP_PWD < WifiPassword > | Parameters: The parameter WifiPassword must be 8-20 characters in length and may contain only letters, numbers, and the special characters '_' and '-'. |
| --- | --- |
| Response Structure SOFTAP_PWD < WifiPassword > |  |
| Command Example |  |
| SET SOFTAP_PWD 88888888 |  |
| Response Example |  |
| SOFTAP_PWD 88888888 |  |
| Additional Notes: Default value: 12345678 |  |

## 6.15 Get Wi-Fi Password

Retrieve the password for the built-in soft AP

| Command Structure GET SOFtAP_PWD | Parameters: |
| --- | --- |
| Response Structure SOFTAP_PWD < WifiPassword > |  |
| Command Example |  |
| GET SOFtAP_PWD |  |
| Response Example |  |
| SOFTAP_PWD 88888888 |  |
| Additional Notes: |  |

## 6.16 Configure Soft AP Function

The built-in Soft AP can be turned on or off. This API controls that.

| Command Structure SET SOFTAP_EN < ON ¦ OFF > | Parameters: ON = Enable Soft AP |
| --- | --- |
| Response Structure SOFTAP_EN < ON ¦ OFF > |  |
| Command Example |  |
| SET SOFTAP_EN OFF |  |
| Response Example |  |
| SOFTAP_EN OFF |  |
| Additional Notes: Default: ON |  |

## 6.17 Get Soft AP On/Off Status

| Command Structure GET SOFTAP_EN | Parameters: |
| --- | --- |
| Response Structure SOFTAP_EN < ON ¦ OFF > |  |
| Command Example |  |
| GET SOFTAP_EN |  |
| Response Example |  |
| SOFTAP_EN OFF |  |
| Additional Notes: |  |

## 6.18 Configure SoftAP Routing

The device supports soft routing, which lets devices connected to the soft AP access network resources on the device's wired LAN port. If that network has internet access, devices on the soft AP can also access the internet.
This API is used to control that feature.

| Command Structure SET SOFTAP_ROUTER < ON ¦ OFF > | Parameters: ON = Enable soft routing |
| --- | --- |
| Response Structure SOFTAP_ROUTER < ON ¦ OFF > |  |
| Command Example |  |
| SET SOFTAP_ROUTER OFF |  |
| Response Example |  |
| SOFTAP_ROUTER OFF |  |
| Additional Notes: Default: ON |  |

## 6.19 Get Soft Router Function Switch Status

| Command Structure GET SOFTAP_ROUTER | Parameters: |
| --- | --- |
| Response Structure SOFTAP_ROUTER < ON ¦ OFF > |  |
| Command Example |  |
| GET SOFTAP_ROUTER |  |
| Response Example |  |
| SOFTAP_ROUTER OFF |  |
| Additional Notes: |  |

## 6.20 Set BYOD Feature

The Miracast and AirPlay receiver functions can be enabled or disabled. This API controls those settings.

| Command Structure SET BYOD_EN < ON ¦ OFF > | Parameters: ON = Enable the BYOD receiver function |
| --- | --- |
| Response Structure BYOD_EN < ON ¦ OFF > |  |
| Command Example |  |
| SET BYOD_EN OFF |  |
| Response Example |  |
| BYOD_EN OFF |  |
| Additional Notes: Default value: ON |  |

## 6.21 Get BYOD Feature

| Command Structure GET BYOD_EN | Parameters: |
| --- | --- |
| Response Structure BYOD_EN < ON ¦ OFF > |  |
| Command Example |  |
| GET BYOD_EN |  |
| Response Example |  |
| BYOD_EN OFF |  |
| Additional Notes: |  |

## 6.22 Set BYOD Access Code

When using Miracast or AirPlay for screen casting, you can require users to enter an access code to prevent interference.
This API controls that feature.

| Command Structure SET BYOD_ACCESS_CODE < AccessCode ¦ AUTO ¦ NONE > | Parameters: AccessCode = Access code; must be 4 digits AUTO = Automatically generate an access code that changes dynamically NONE = No access code |
| --- | --- |
| Response Structure BYOD_ACCESS_CODE < AccessCode ¦ AUTO ¦ NONE > |  |
| Command Example |  |
| SET BYOD_ACCESS_CODE 1234 |  |
| Response Example |  |
| BYOD_ACCESS_CODE 1234 |  |
| Additional Notes: Default: NONE |  |

## 6.23 Get BYOD Access Code

| Command Structure GET BYOD_ACCESS_CODE | Parameters: Optional parameter AUTO indicates the current access code is dynamically generated. |
| --- | --- |
| Response Structure BYOD_ACCESS_CODE NONE 或 BYOD_ACCESS_CODE < AccessCode > [ AUTO ] |  |
| Command Example |  |
| GET BYOD_ACCESS_CODE |  |
| Response Example |  |
| BYOD_ACCESS_CODE 1234 AUTO |  |
| Additional Notes: |  |

## 6.24 Set SSID Broadcast

By default, a soft AP periodically broadcasts packets that include its SSID. Disabling this feature can improve security to some extent. This API is used to control that behavior.

| Command Structure SET SSID_BROADCAST_FN < ON ¦ OFF > | Parameters: ON = SSID broadcasting enabled |
| --- | --- |
| Response Structure SSID_BROADCAST_FN < ON ¦ OFF > |  |
| Command Example |  |
| SET SSID_BROADCAST_FN OFF |  |
| Response Example |  |
| SSID_BROADCAST_FN OFF |  |
| Additional Notes: Default: ON |  |

## 6.25 Get SSID Broadcast Switch Status

| Command Structure GET SSID_BROADCAST_FN | Parameters: |
| --- | --- |
| Response Structure SSID_BROADCAST_FN < ON ¦ OFF > |  |
| Command Example |  |
| GET SSID_BROADCAST_FN |  |
| Response Example |  |
| SSID_BROADCAST_FN OFF |  |
| Additional Notes: |  |

Publication Disclaimer:

The material contained in this document consists of information that is the sole property of WyreStorm. This document is intended to provide information to allow interfacing to the relevant WyreStorm equipment by third party products

WYRESTORM IS NOT RESPONSIBLE FOR MALFUNCTIONS AND/OR THE IN-OPERABILITY WHICH MAY BE CAUSED BY THE APPLICATION OF THIS INFORMATION, WHETER EXPECTED OR NOT.

WyreStorm reserves the right to change software, control codes and specifications without notice.

WyreStorm will not be liable for any use of this information or any changes it may make to these products. The use of this information constitutes an agreement by the user to these limitations and exclusions.

WyreStorm Offices

North America: 27 Wood Road, Round Lake NY

12151 Tel: +1 518-289-1293

EMEA: Unit 22, Ergo Business Park, Swindon, Wiltshire, SN3

3JW, UK Tel: +44 (0) 1793230343

WyreStorm Technologies reserves the right to change physical appearance or technical specification of this product at any time. Visit wyrestorm.com for the latest product information.